./

GENESIS – Networking Learning Report

| Ver. Rel. No. | Release Date | Prepared. By | Reviewed By | To be Approved | Remarks/Revision Details |
| --- | --- | --- | --- | --- | --- |
| 1.0 | 21-12-20 | - Megha Annie George |  | Srinivas K |  |

Details

Contents

List of Figures	4

List of Tables	4

1	NETWORKING CONCEPTS	5

1.1	NETWORK TOPOLOGIES	5

1.1.1	MESH TOPOLOGY	5

1.1.2	STAR TOPOLOGY	6

1.1.3	BUS TOPOLOGY	6

1.1.4	RING TOPOLOGY	7

1.1.5	TREE TOPOLOGY	8

1.1.6	HYBRID TOPOLOGY	8

1.2	WIRED AND WIRELESS NETWORKS	9

1.2.1	PAN	10

1.2.2	LAN	11

1.2.3	MAN	11

1.2.4	WAN	11

1.2.5	WLAN	12

1.2.6	WI-FI	12

1.2.7	WIMAX	12

2	COMPONENTS	13

2.1	HUB	13

2.2	SWITCH	14

2.3	ROUTER	14

2.4	BRIDGE	14

2.5	GATEWAY	15

2.6	WIRELESS ACCESS POINTS	15

3	PROTOCOLS	16

3.1	OSI MODEL	16

3.2	TCP	17

3.3	UDP	19

3.4	INTERNET PROTOCOL	20

3.5	L2 PROTOCOLS	21

3.6	L3 PROTOCOLS	22

4	IP ADDRESSES	23

4.1	IPV4 V/S IPV6	23

4.2	NETWORK CLASSES IN IPV4	25

4.2.1	CLASS A	25

4.2.2	CLASS B	25

4.2.3	CLASS C	25

4.3	SUBNETTING	26

5	COLLEGE NETWORK USING CISCO PACKET TRACER	28

# List of Figures

Figure 1 MESH TOPOLOGY	5

Figure 2 STAR TOPOLOGY	6

Figure 3 BUS TOPOLOGY	6

Figure 4 RING TOPOLOGY	7

Figure 5 TREE TOPOLOGY	8

Figure 6 HYBRID TOPOLOGY	9

Figure 7 NETWORK HIERARCHY	10

Figure 8 PAN NETWORK	11

Figure 9 LAN NETWORK	11

Figure 10 MAN NETWORK	12

Figure 11 WAN NETWORK	12

Figure 12 NETWORK COMPONENTS HIERARCHY	13

Figure 13 HUB	14

Figure 14 SWITCH	14

Figure 15 ROUTER	14

Figure 16 BRIDGE	15

Figure 17 GATEWAY	15

Figure 18 WIRELESS ACCESS POINT	16

Figure 19 OSI MODEL	16

Figure 20 OSI MODEL - DATA UNIT	17

Figure 21 TCP	17

Figure 22 DATA FLOW IN TCP	18

Figure 23 TCP HEADER	18

Figure 24 UDP	19

Figure 25 DATA FLOW IN UDP	19

Figure 26 UDP HEADER	20

Figure 27 DATA FLOW IN INTERNET PROTOCOL SUITE	20

Figure 28 IP HEADER	21

Figure 29 IPV4 V/S IPV6	23

Figure 30 DIFFERENCE BETWEEN IPV4 & IPV6	24

Figure 31 IPV4 & IPV6 HEADER	24

Figure 32 NETWORK CLASSES	26

Figure 33 NETWORK CLASS MASK	26

Figure 34 COLLEGE NETWORK DESIGN	27

Figure 35 PING SCREENSHOT	28

# List of Tables

Table 1 MESH TOPOLOGY	5

Table 2 STAR TOPOLOGY	6

Table 3 BUS TOPOLOGY	7

Table 4 RING TOPOLOGY	8

Table 5 TREE TOPOLOGY	8

Table 6 HYBRID TOPOLOGY	9

Table 7 NETWORK TYPE	10

Table 8 L2 PROTOCOLS	21

Table 9 L3 PROTOCOLS	22

# NETWORKING CONCEPTS

## NETWORK TOPOLOGIES

The arrangement of a network which comprises of nodes and connecting lines via sender and receiver is referred as network topology.

### MESH TOPOLOGY

In mesh topology, every device is connected to another device via a particular channel.

Figure 1 MESH TOPOLOGY

Table 1 MESH TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - It is robust. - Fault is diagnosed easily. Data is reliable because data is transferred among the devices through dedicated channels or links. - Provides security and privacy. | - Installation and configuration is difficult. - Cost of cables are high as bulk wiring is required, hence suitable for less number of devices. - Cost of maintenance is high. |

### STAR TOPOLOGY

In star topology, all the devices are connected to a single hub through a cable. This hub is the central node and all other nodes are connected to the central node.

Figure 2 STAR TOPOLOGY

Table 2 STAR TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - If N devices are connected to each other in star topology, then the number of cables required to connect them is N. So, it is easy to set up. - Each device require only 1 port i.e. to connect to the hub. | - If the concentrator (hub) on which the whole topology relies fails, the whole system will crash down. - Cost of installation is high. - Performance is based on the single concentrator i.e. hub. |

### BUS TOPOLOGY

Bus topology is a network type in which every computer and network device is connected to single cable. No bi-directional feature is in bus topology.

Figure 3 BUS TOPOLOGY

Table 3 BUS TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - If N devices are connected to each other in bus topology, then the number of cables required to connect them is 1 which is known as backbone cable and N drop lines are required. - Cost of the cable is less as compared to other topology, but it is used to built small networks. | - If the common cable fails, then the whole system will crash down. - If the network traffic is heavy, it increases collisions in the network. To avoid this, various protocols are used in MAC layer known as Pure Aloha, Slotted Aloha, CSMA/CD etc. |

### RING TOPOLOGY

In this topology, it forms a ring connecting devices with its exactly two neighboring devices. The transmission is unidirectional, but it can be made bidirectional by having 2 connections between each Network Node, it is called Dual Ring Topology.

Figure 4 RING TOPOLOGY

Table 4 RING TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - The possibility of collision is minimum in this type of topology. - Cheap to install and expand. | - Troubleshooting is difficult in this topology. - Addition of stations in between or removal of stations can disturb the whole topology. |

### TREE TOPOLOGY

This topology is the variation of Star topology. This topology has hierarchical flow of data.

Figure 5 TREE TOPOLOGY

Table 5 TREE TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - It allows more devices to be attached to a single central hub thus it increases the distance that is travel by the signal to come to the devices. - It allows the network to get isolated and to prioritize from different computers. | - If the central hub gets fails the entire system fails. - The cost is high because of cabling. |

### HYBRID TOPOLOGY

Hybrid Topology is basically a network topology comprising of two or more different types of topologies.

Figure 6 HYBRID TOPOLOGY

Table 6 HYBRID TOPOLOGY

| ADVANTAGES | DISADVANTAGES |
| --- | --- |
| - It is easy to troubleshoot and provides simple error detecting techniques. - It is a flexible network topology, making it quite effective. - It is scalable since the size can be made greater easily. | - If the central hub gets fails the entire system fails. - The cost is high because of cabling. |

## WIRED AND WIRELESS NETWORKS

Figure 7 NETWORK HIERARCHY

Table 7 NETWORK TYPE

| NETWORK | SPEED | RANGE | EXAMPLE |
| --- | --- | --- | --- |
| PAN | 250 Kbps – 24 Mbps | < 10 m | Bluetooth |
| LAN | 100 Mbps / 1 Gbps / 10 Gbps | < 1 Km | Wi-Fi |
| MAN | ~ 50 Mbps | 50 – 100 Km | WiMAX |
| WAN | 20 Mbps / 50 Mbps / 100Mbps | 100 + Km | GSM, GPRS |

### PAN

PAN is a computer network formed around a person. It generally consists of a computer, mobile, or personal digital assistant. PAN can be used for establishing communication among these personal devices for connecting to a digital network and the internet.

Figure 8 PAN NETWORK

### LAN

A Local Area Network (LAN) is a group of computer and peripheral devices which are connected in a limited area such as school, laboratory, home, and office building. It is a widely useful network for sharing resources like files, printers, games, and other application.

Figure 9 LAN NETWORK

### MAN

A Metropolitan Area Network or MAN is consisting of a computer network across an entire city, college campus, or a small region. Depending upon the type of configuration, this type of network allows you to cover an area from several miles to tens of miles.

Figure 10 MAN NETWORK

### WAN

WAN (Wide Area Network) is another important computer network that which is spread across a large geographical area. WAN network system could be a connection of a LAN which connects with other LAN's using telephone lines and radio waves. It is mostly limited to an enterprise or an organization.

Figure 11 WAN NETWORK

### WLAN

WLAN (Wireless Local Area Network) helps you to link single or multiple devices using wireless communication within a limited area like home, school, or office building. It gives users an ability to move around within a local coverage area which may be connected to the network. Today most modern day's WLAN systems are based on IEEE 802.11 standards.

### WI-FI

WI-FI is a family of wireless network protocols, based on the IEEE 802.11 family of standards, which are commonly used for local area networking of devices and Internet access.

### WIMAX

Acronym for Worldwide Interoperability for Microwave Access. Based on Wireless MAN technology. A wireless technology optimized for the delivery of IP centric services over a wide area. A scalable wireless platform for constructing alternative and complementary broadband networks. A certification that denotes interoperability of equipment built to the IEEE 802.16 or compatible standard.

# COMPONENTS

Figure 12 NETWORK COMPONENTS HIERARCHY

## HUB

A Hub is a hardware device that divides the network connection among multiple devices. When computer requests for some information from a network, it first sends the request to the Hub through cable. Hub will broadcast this request to the entire network. All the devices will check whether the request belongs to them or not. If not, the request will be dropped. Hub consumes more bandwidth.

Figure 13 HUB

## SWITCH

A switch is a hardware device that connects multiple devices on a computer network. The Switch contains the updated table that decides where the data is transmitted or not. Switch delivers the message to the correct destination based on the physical address present in the incoming message. A Switch does not broadcast the message to the entire network like the Hub. It increases the speed of the network.

Figure 14 SWITCH

## ROUTER

A router is a hardware device which is used to connect a LAN with an internet connection. It is used to receive, analyze and forward the incoming packets to another network based on the information available in the routing table. A router works in a Layer 3 (Network layer) of the OSI Reference model. It determines the best path from the available paths for the transmission of the packet.

Figure 15 ROUTER

## BRIDGE

A bridge interconnects two networks using the same technology (such as Ethernet or Arc net). A modern bridge reads the destination address of the received packet and determines whether the address is on the same segment of the network cables of the originating station. If the destination is on the other side of the bridge, the bridge transmits the packet into the traffic on that cable segment. Local bridges are used to connect two segments of the same LAN. Remote bridges are used to link local LAN cables to thin long-distance cables to link two physically separated network. Bridges are easy to install.

Figure 16 BRIDGE

## GATEWAY

Two different networks can be connected using a gateway. For example, a mainframe can be connected and accessible to a PC network using a gateway. Unlike routers, a gateway converts the format of the data sent between two networks. A router adds only addressing information to the data packet. Routers never change the content of the message. However, a gateway identifies the protocols used in the networks, and recognize the data format and convert the message format into a suitable format to be accepted by the other network. Wide area networks often use gateways because there is many different networks present in a WAN.

Figure 17 GATEWAY

## WIRELESS ACCESS POINTS

In computer networking, a wireless access point (WAP), or more generally just access point (AP), is a networking hardware device that allows other Wi-Fi devices to connect to a wired network. The AP connects to a router (via a wired network) as a standalone device, but it can also be an integral component of the router itself. An AP is differentiated from a hotspot which is a physical location where Wi-Fi access is available.

Figure 18 WIRELESS ACCESS POINT

# PROTOCOLS

## OSI MODEL

OSI stands for Open Systems Interconnection. It is a 7-layer architecture with each layer having specific functionality to perform. All these 7 layers work collaboratively to transmit the data from one person to another across the globe.

Figure 19 OSI MODEL

Figure 20 OSI MODEL - DATA UNIT

## TCP

Figure 21 TCP

Figure 22 DATA FLOW IN TCP

Figure 23 TCP HEADER

## UDP

Figure 24 UDP

Figure 25 DATA FLOW IN UDP

Figure 26 UDP HEADER

## INTERNET PROTOCOL

The Internet Protocol (IP) is the principal communications protocol in the Internet protocol suite for relaying datagrams across network boundaries. IP has the task of delivering packets from the source host to the destination host solely based on the IP addresses in the packet headers. It also defines addressing methods that are used to label the datagram with source and destination information.

Figure 27 DATA FLOW IN INTERNET PROTOCOL SUITE

Figure 28 IP HEADER

## L2 PROTOCOLS

Layer 2 contains two sublayers:

- Logical link control (LLC) sublayer, which is responsible for managing communications links and handling frame traffic.

- Media access control (MAC) sublayer, which governs protocol access to the physical network medium. By using the MAC addresses that are assigned to all ports on a switch, multiple devices on the same physical link can uniquely identify one another.

Table 8 L2 PROTOCOLS

| PROTOCOL | DESCRIPTION |
| --- | --- |
| LLDP (Link layer discovery protocol) | LLDP is vendor neutral, and is commonly used as a component in network management and network monitoring applications. |
| CDP (Cisco Discovery Protocol) | CDP is a Cisco proprietary protocol that support the IEEE 802.1ab version of LLDP, and is primarily used to share information between directly connected Cisco devices. |
| IP route | This command contains information from the IP routing table that can be used to forward a packet through the best path towards its destination. |
| FDB (Forwarding database) | FDB stores MAC addresses of the discovered devices and their respective ports. This protocol is preferred for discovering switches. |
| ARP (Address Resolution Protocol) | ARP maps dynamic IP (Layer 3) with MAC addresses (Layer 2). ARP translates 32-bit addresses to 48-bit and vice versa, and is preferred by IPv4 devices. |
| MLT (Multi-link trucking Protocol) | MLT provides high-speed, fault tolerant connection between servers, switches and routers by grouping all ethernet links into a single logical ethernet link. |
| CAN (Controller area network) | CAN facilitates communication between the applications of microcontrollers and their devices without relying on a host computer. |
| PPP (Point-to-Point Protocol) | PPP allows you to establish communication between two routers without the help of a host. |

## L3 PROTOCOLS

L3(Layer3) does routing, logical addressing, packet sequencing and provide interfacing with TCP or UDP.

Table 9 L3 PROTOCOLS

| PROTOCOL | DESCRIPTION |
| --- | --- |
| CLNP (Connectionless Networking Protocol) | Connectionless Network Protocol is an OSI Network Layer datagram service that does not require a circuit to be established before data is transmitted, and routes messages to their destinations independently of any other messages. |
| NAT (Network Address Translation) | Network Address Translation (NAT) is a process in which one or more local IP address is translated into one or more Global IP address and vice versa in order to provide Internet access to the local hosts. |
| ICMP (Internet Control Message Protocol) | It is used for reporting errors and management queries. It is a supporting protocol and used by networks devices like routers for sending the error messages and operations information. |
| OSPF (Open Shortest Path First (v1 and v2)) | Open Shortest Path First (OSPF) is a link-state routing protocol that is used to find the best path between the source and the destination router using its own Shortest Path First |
| RIP (Routing Information Protocol (v1 and v2)) | Routing Information Protocol (RIP) is a dynamic routing protocol which uses hop count as a routing metric to find the best path between the source and the destination network. |

# IP ADDRESSES

## IPV4 V/S IPV6

Figure 29 IPV4 V/S IPV6

Figure 30 DIFFERENCE BETWEEN IPV4 & IPV6

Figure 31 IPV4 & IPV6 HEADER

## NETWORK CLASSES IN IPV4

The first step in planning for IP addressing on your network is to determine which network class is appropriate for your network. After you have done this, you can take the crucial second step: obtain the network number from the InterNIC addressing authority.

Currently there are three classes of TCP/IP networks. Each class uses the 32-bit IP address space differently, providing more or fewer bits for the network part of the address. These classes are class A, class B, and class C.

### CLASS A

A class A network number uses the first eight bits of the IP address as its "network part." The remaining 24 bits comprise the host part of the IP address. The values assigned to the first byte of class A network numbers fall within the range 0-127. Consider the IP address 75.4.10.4. The value 75 in the first byte indicates that the host is on a class A network. The remaining bytes, 4.10.4, establish the host address. The InterNIC assigns only the first byte of a class A number. Use of the remaining three bytes is left to the discretion of the owner of the network number. Only 127 class A networks can exist. Each one of these numbers can accommodate up to 16,777,214 hosts.

### CLASS B

A class B network number uses 16 bits for the network number and 16 bits for host numbers. The first byte of a class B network number is in the range 128-191. In the number 129.144.50.56, the first two bytes, 129.144, are assigned by the InterNIC, and comprise the network address. The last two bytes, 50.56, make up the host address, and are assigned at the discretion of the owner of the network number. Class B is typically assigned to organizations with many hosts on their networks.

### CLASS C

Class C network numbers use 24 bits for the network number and 8 bits for host numbers. Class C network numbers are appropriate for networks with few hosts--the maximum being 254. A class C network number occupies the first three bytes of an IP address. Only the fourth byte is assigned at the discretion of the network owners. The first byte of a class C network number covers the range 192-223. The second and third each cover the range 1- 255. A typical class C address might be 192.5.2.5. The first three bytes, 192.5.2, form the network number. The final byte in this example, 5, is the host number.

Figure 32 NETWORK CLASSES

Figure 33 NETWORK CLASS MASK

## SUBNETTING

Subnetting is the process of breaking down a single IP address block into smaller subnetworks (subnets). The reason we need to subnet is to efficiently distribute IP addresses with the result of less wastage.

- Step 1 – determining an appropriate class of address and why

Using the formula 2H, where H represents the host bit, we get the following results:

Class A = 224 = 16,777,216 total IPs

Class B = 216 = 65,536 total IPs

Class C = 28 = 256 total IPs

These are the Network ID and the Broadcast IP address. Therefore, you need to subtract two addresses from the total IP formula.

Using the formula 2H-2 to calculate usable IPs, we get the following:

Class A = 224 – 2 = 16,777,214 total IPs

Class B = 216 – 2 = 65,534 total IPs

Class C = 28 – 2 = 254 total IPs

- Step 2 – creating subnets (subnetworks)

To create more subnets or subnetworks, we need to borrow bits on the host portion of the network. The formula 2N is used to calculate the number of subnets, where N is the number of bits borrowed on the host portion. Once these bits are borrowed, they will become part of the network portion and a new subnet mask will be presented.

- Step 3 – assigning each network an appropriate subnet and calculating the ranges

To determine the first usable IP address within a subnet, the first bit from the right must be 1. To determine the last usable IP address within a subnet all of the host bits except the first bit from the right should all be 1s. The broadcast IP of any subnet is when all of the host bits are 1s.

- Step 4 – VLSM and subnetting a subnet

For the WAN links, we need at least three subnets. Each must have a minimum of two usable IP addresses. To get started, let’s use the following formula to determine the number of host bits that are needed so that we have at least two usable IP addresses: 2H – 2, where H is the number of host bits.

We are going to use one bit, 21 – 2 = 2 – 2 = 0 usable IP addresses. Let’s add an extra host bit in our formula, that is, 22 – 2 = 4 – 2 = 2 usable IP addresses. At this point, we have a perfect match, and we know that only two host bits are needed to give us our WAN (point-to-point) links.

# COLLEGE NETWORK USING CISCO PACKET TRACER

Figure 34 COLLEGE NETWORK DESIGN

Figure 35 PING SCREENSHOT